KD800 Touch Screen Digital lock for Cabinet/ Locker

Features:

- Secure control by level-management;

- Touch screen Digital with EM card type;

- Permanent code or one-time code convertible;

- Indicator blinks while in locked status;

- Wrong code or illegal card alarm;

- Anti-picked (prized) alarm;

- Low battery alarm;

- External power back up;

- ABS plastic material;

Technical Parameters

| Power supply | 3 x AA alkaline batteries |
| --- | --- |
| Static current | ≤30µA |
| Dynamic current | ≤200mA |
| Working temperature | -15 ～ +55℃ |
| Working humidity | 20 ～ 95%RH |
| Alarm voltage | 3.5±0.2V |

Emergency unlocking

When the batteries are exhausted and it is locked, use a 9V battery charger to connect with the 2 contacts and enter password or scan card to unlock.

Latch mechanism: 300,000 life times (motor life times:450,000 times);Powered by 3 pcs AA alkaline batteries and batteries last at least 12 months.

Project Case

Package:

2 pcs/inner box

Carton size: 430mm x 270mm x 220mm

40 pcs/ctn (7.50kg)

FAQ

Q:Can this password locker lock be used for public?
A:Yes, you can use it to public, use guest code to close it, then same code to open it.

Q:Is master card available for the digital locker lock?
A:It is available, it will be provided master card to open the lock for emergency.

Q:If user forgot password, how to open the locker?
A:Use master card to unlock it.

Q:What can we do if the power is exhausted.
A:There is consecutive 8 sounds as alarm from the locks, and please replace the batteries after sound alarm.
If the batteries are dead in the locker lock, use external power to open it and change batteries.